🌾 Final Design Summary — Makhana Seed Collector (Hybrid Bamboo Basket Model)

1️⃣ User & Context

| Item | Detail |
| --- | --- |
| Primary users | Adult male makhana seed collectors (standing posture) |
| Field environment | Shallow pond beds, mud, vegetation, repeated pulling & shaking |
| Task | Pull tool → collect seeds → lift basket from holder → shake/sieve to remove water/mud |
| Design goal | Reduce bending & fatigue, ensure easy pulling and shaking, integrate traditional bamboo design with durable frame |

2️⃣ Anthropometric Reference (Males, Indian population)

| Parameter | 5th | 50th | 95th | Units |
| --- | --- | --- | --- | --- |
| Elbow height | 945 | 1039 | 1123 | mm |
| Knee height | 419 | 464 | 517 | mm |
| Forward reach | 749 | 849 | 939 | mm |
| Shoulder breadth | 341 | 380 | 422 | mm |
| Hand length | 163 | 180 | 198 | mm |
| Grip diameter (inside) | 42 | 49 | 56 | mm |

3️⃣ Key Ergonomic Heights (final)

| Feature | Dimension (mm) | Notes |
| --- | --- | --- |
| Handle pivot height (Hₚ) | 700 ± 30 | Grip ≈ elbow height at 25° working angle |
| Handle length (L) | 800 | Comfortable reach vs torque |
| Knee-stop angle (θₖ) | −20° ± 2° | Handle rests near knee height |
| Holder (basket seat) height (H_holder) | 780 ± 30 | Ergonomic lift starting point |
| Shake height (H_shake) | ≈ 1030 ± 100 | At/near elbow height for average male |
| Lift distance (Δlift) | ~250 | From seat to shake position |
| Clearance above shake | ≥ 350 | For seed fall and hand movement |

4️⃣ Basket Geometry (updated for 780 mm holder height)

| Property | Value | Notes |
| --- | --- | --- |
| Usable volume | ≈ 45 L (0.045 m³) | Holds ~15 kg raw seeds |
| Gross volume | ≈ 50–55 L | Allows 10–20 % extra for water/mud |
| Internal dimensions | W = 500 mm × L = 300 mm × H = 300 mm | 45 L rectangular form |
| Outer dimensions | 503×303×303 mm (1.5 mm wall) | Adjust per sheet thickness |
| Rim height (from ground in holder) | ≈ 780 mm | Aligns with ergonomic lift height |
| Grip handles above rim | +30 mm | Grip ≈ 810 mm when seated |
| Lift to shake | ~220 mm → grip ≈ 1030 mm (perfect ergonomics) |  |

5️⃣ Basket Construction — Hybrid (Traditional + Modern)

| Component | Material / Spec | Function |
| --- | --- | --- |
| Outer shell | Hand-woven bamboo strips, 2–3 mm thick, 10–15 mm weave pitch | Traditional aesthetic, lightweight body |
| Rim | MS flat 25×4 mm, galvanized or SS 304 20×3 mm | Maintains shape, provides bolt points |
| Inner false bottom | SS 304 mesh, 2–3 mm aperture (Ø0.8–1 mm wire) or SS 304 plate 1.5 mm Ø3.5 mm holes @ 10 mm pitch | Drainage + seed retention |
| Removable liner | PP/nylon mesh bag, 1–2 mm aperture | Retains seeds, easy cleaning |
| Frame support | MS tube 30×20×2 mm, galvanized | Transfers load to chassis |
| Fasteners | SS M8 bolts, nylock nuts, rubber washers + internal 50×30×3 mm backing plates | Prevents bamboo cracking |
| Finish | Bamboo treated with borax/boric acid + 2 coats marine varnish | Moisture & rot protection |

Expected empty mass: 6–9 kg
Full mass (with seeds): 20–25 kg
Design load: ≥ 30 kg static (FOS ≥ 1.5)

6️⃣ Loader Bucket + Brush

| Element | Specification | Notes |
| --- | --- | --- |
| Loader bucket | 500 W × 300 H × 300 L mm (≈ 45 L) | Matches basket; mild-steel or SS perforated base |
| Lip clearance | 10–20 mm above ground | Avoid digging |
| Front tines | 25–30 mm spacing | Lets water pass, traps seeds |
| Brush width | 500 mm (match bucket) | Even sweep |
| Bristle length | 60–80 mm, nylon/PP | Stiff enough for pond bed |
| Brush shaft | Ø 16–20 mm steel, slow-rotating | Corrosion-protected bearings |

7️⃣ Wheels & Chassis

| Parameter | Specification |
| --- | --- |
| Wheel Ø | 200–250 mm (250 mm ideal) |
| Tread width | 60–80 mm wide |
| Type | Pneumatic or solid rubber, deep tread |
| Axle Ø | 12–16 mm steel, greaseable bearings |
| Wheel load rating | ≥ 500 N each |
| Frame material | Galv. MS 30×20×2 mm tubing |
| Finish | Hot-dip galvanized or powder-coated |

8️⃣ Ergonomic Pull & Torque Forces

| Condition | Total Mass | C_rr | Pull Force (N) | Handle Moment (N·m @ 0.8 m) |
| --- | --- | --- | --- | --- |
| Normal mud | 18 kg | 0.20 | 35 N | 28 N·m |
| Sticky mud | 23 kg | 0.30 | 68 N | 54 N·m |
| Design limit | — | — | 400 N peak | 320 N·m max |

- Comfortable sustained pull: 50–100 N

- Short-term peak: 150–250 N

- Basket + frame safe for 25–30 kg static, 400 N dynamic pull (FOS ≈ 2).

9️⃣ Coverage Before Emptying

Assuming basket = 15 kg, efficiency E = 0.6, sweep = 0.5 m:

| Field Yield (kg/ha) | Area per fill (m²) | Sweep length (m) |
| --- | --- | --- |
| 1200 (low) | 208 | 417 |
| 2000 (typical) | 125 | 250 |
| 3000 (high) | 83 | 167 |

≈ 100–150 m² typical coverage per fill (≈ 16 k seeds).

🔟 Shaking / Sifting Stage Heights (ergonomic)

| Parameter | Dimension (mm) | Notes |
| --- | --- | --- |
| Holder (top of seat) | 780 ± 30 | Comfortable start height |
| Grip while seated | ≈ 810 mm | Rim + handle offset |
| Shake height | ≈ 1030 ± 100 | Elbow-level |
| Lift distance | 220–250 mm | Ergonomic small lift |
| Clearance above | ≥ 350 mm | Safe shaking space |
| Two-hand handle span | 300–350 mm | Secure grip |
| Max lift mass | 12–15 kg (single person) | Recommend 2-handed lift when full |

1️⃣1️⃣ Safety / Maintenance Checklist

- ✅ Mark max load (≤ 15 kg) on basket.

- ✅ Reinforce rim with steel ring; do not hang load from bamboo alone.

- ✅ Drain holes or sloped mesh base for quick drying.

- ✅ Grease bearings, inspect bamboo for splits each season.

- ✅ Replace liner if torn or clogged.

- ✅ Store basket dry and out of direct sun after use.

- ✅ Annual varnish or oil coating on bamboo.

1️⃣2️⃣ Optional Further Calculations / Refinements

| Aspect | Why it matters | Simple Method |
| --- | --- | --- |
| Center of Gravity | Stability, balance when full | CAD mass model |
| Handle tube stress | Confirm bending yield | σ = Mc/I (for 320 N·m moment) |
| Frame deflection | Avoid wobble | Beam calc on 30×20×2 mm MS |
| Efficiency (E) | Determines area coverage | Field trial (mass collected vs area) |
| Drain rate | Prevent water overload | Fill test → time to empty |

✅ Quick CAD / Fabrication Reference

| Item | Dimension (mm) | Note |
| --- | --- | --- |
| Basket internal | 500 × 300 × 300 | ≈ 45 L usable |
| Basket rim height | 780 | In seated holder |
| Handle pivot height | 700 | Comfortable pull |
| Handle length | 800 | Ergonomic reach |
| Brush width | 500 | Matches basket |
| Wheel Ø / width | 250 / 70 | Mud performance |
| Axle Ø | 14 | Bearing strength |
| Overall product height | ≈ 950–1000 | Includes rim + holder frame + wheels |
| Overall width (outer) | ≤ 480 | Below 95th shoulder (422 mm + margin) |

So your final product height from ground to top of seated basket = ~950–1000 mm (including wheel diameter + frame).
When lifted for shaking, basket rim reaches ≈ 1030–1050 mm, perfectly at elbow height — ergonomic and safe.

🌿 Final Verdict

Ergonomic + Structural Harmony:

A 45 L hybrid bamboo–metal basket mounted at ~780 mm seat height, lifted ~250 mm to shake (~1030 mm), combined with 800 mm handle & 250 mm wheels, yields an overall height ≈ 1 m, easy pulling (< 70 N) and comfortable shaking without back or shoulder strain.

Would you like me to make a formatted one-page “Final Design Specification Sheet” (clean layout, ready for submission or fabrication) using this data next?